## General Rules and Safe ESP32 Download/Flashing Guide

Designed to be easy to understand, safe to follow, and suitable as an official reference.
This guide also highlights critical points that are often overlooked.

## General Rules for ESP32 Egg Incubator Firmware

### 1️⃣ Version & Board Information

- Firmware: Demo Version 2.0.3 (Full version via contact: 0813-1970-7919)

- Build Date: February 20, 2024

- Board: ESP32-DEV-MODULE

- Flash Size: 4MB (32Mb)

- Partition Scheme: Huge APP (3MB No OTA + 1MB LittleFS)

- Crystal: 40MHz

- SPI Flash: DIO, 80MHz

- Arduino Core Version: 2.0.17

### 2️⃣ Required Libraries

The firmware uses the following libraries:

- Adafruit Bus IO 1.17.4

- Adafruit Unified Sensor 1.1.15

- ArduinoJson 7.4.2

- AsyncTCP 3.4.10

- DHT Sensor Library 3.4.6

- ESPAsyncWebServer 3.9.5

- MD_MAX72XX 3.5.1

- MD_Parola 3.7.5

- RTClib 2.1.4

⚠️ Ensure the library versions match exactly for full compatibility.

### 3️⃣ Default ESP32 Pinout

| Function | Pin |
| --- | --- |
| Temperature & Humidity Sensor |  |
| DHT22 PIN | 4 |
| RTC DS3231 |  |
| SDA | 21 |
| SCL | 22 |
| Dot Matrix LED Display |  |
| CLK_PIN | 18 |
| CS_PIN | 5 |
| DATA_PIN | 23 |
| Incubator Components |  |
| Buzzer | 26 |
| Heater | 27 |
| Fan | 19 |
| Humidifier | 17 |
| Egg Turner | 16 |
| Built-in LED (NC) | 2 |

### 4️⃣ Safe ESP32 Flashing Rules

#### 1. Preparation

- Use ESP32 Flash Download Tool v3.9.9

- Connect ESP32 to PC via USB

- Ensure the correct COM port

- Backup LittleFS if you want to keep data from previous firmware

#### 2. Safe Flashing Order

- Erase Flash (only when changing from old firmware)

- Flash bootloader

- Flash partition table

- Flash firmware (.bin)

- Flash LittleFS (.bin)

⚠️ Important Note:
LittleFS must always be at address 0x310000 for the huge_app scheme.
Do not change the flashing order or addresses.

## 3. Flash Download Tool Layout

### Initial Mode

- Chip Type: ESP32

- Work Mode: Develop

### File List

| File | Address |
| --- | --- |
| EggIncubator_ESP32_V203_DHT22_DS3231_en.ino.bootloader.bin | 0x1000 |
| EggIncubator_ESP32_V203_DHT22_DS3231_en.ino.partitions.bin | 0x8000 |
| EggIncubator_ESP32_V203_DHT22_DS3231_en.ino.bin | 0x10000 |
| EggIncubator_ESP32_V203_DHT22_DS3231_en.ino.littlefs.bin | 0x310000 |

### Flash Settings

- SPI MODE: DIO

- SPI SPEED: 80MHz

- FLASH SIZE: 4MB

- COM: COM5 (adjust according to PC)

- Baudrate: 115200

### Additional Options

- Erase Flash: ✅ enabled (once only)

- DoNotChgBin: ❌ must NOT be checked

## 4. Flashing Process

- Click START in Flash Download Tool

- If not detected: press and hold the BOOT button on ESP32 until flashing starts

- Wait until FINISH / SUCCESS

### Signs of Successful Flashing

- No red error messages

- ESP32 resets automatically

- Web server / LittleFS accessible

- No reboot loop

## 5. Troubleshooting

| Issue | Solution |
| --- | --- |
| Blank web / 404 | Check LittleFS address (must be 0x310000) |
| Continuous reset | Verify firmware & partition, re-flash |
| Cannot connect | Press & hold BOOT during START |
| Slow / failed flash | Lower baudrate to 74880 or 115200 |

## 6. Standard Debug Log

- LittleFS mount success

- RTC init success

- DHT sensor normal

- WiFi connected (STA Mode)

- Dashboard status: registered

## ✅ Key Tips Summary

- Always match SPI mode settings

- LittleFS must be at 0x310000 for huge_app

- Flashing order must not be changed

- Use erase flash once when changing firmware

- Hold BOOT button if ESP32 is not detected

Happy flashing and good luck! 🚀